ENGR-UH 1000

Computer Programming for Engineers

Assignment 4 – Electrical Engineering Case Study Robot Kinematics Software Report

Gulsim Azirakhmet

Fall 2021

Step 1: Problem Identification and Statement

This assignment involves the design of a software to simulate the motion of a multi-degree of freedom kinematic chains that form the structure of a robotic system. The emphasis on geometry means that the links of the robot are modeled as rigid bodies and its joints are assumed to provide pure rotation or translation.

Step 2: Gathering of Information and Input/Output Description

Figure 1: (left) Example of a robot chain with three segments, (right) the parameters of a segment. The system has the following components:

1. A segment: A segment is a rigid member connecting two joints. As shown in Figure 1(right), a segment has the following parameters: a starting point (x1 and y1 coordinates), a length, an angle (different technique may have different references for measuring the angle for a segment, so make your assumption clear), and an end point (x2 and y2 coordinates). Note that the coordinates of the end point can be calculated given the start point coordinates, the length, and the angle.

2. A Robot: A robot is comprised of a chain of segments that are connected via joints. The first segment is called the root as it is connected to the robot base/ground. A robot can have multiple segments, which is commonly known as the degrees of freedom. The end effector of the robot is the end point of the last segment (noted as (x,y) in Figure 1-left). In order to determine the position of the end effector, the position/orientation of all the segments must be determined – which is usually controlled through the angles of the segments. The forward kinematic analysis involves calculating the end effector position for a specific set of angles for the segments. The inverse kinematics analysis involves calculating the angles of all the segments that move the end effector to a particular position. This assignment will focus on the forward kinematics analysis.

In order to create a plot I downloaded pbPlots library. In Google I was able to find and download the library. After adding libraries and linking them, i looked up for the sample function codes to use pbPlots.

Input/Output description:

Coordinates of the last point

number of segments

angle

Graph of the segments

length

root segment length and

angle

Step 3: Design of the algorithm and hand-solved problems

Test cases:

Test Case 1: printS() function

Input:

Angle = 0.8 radians

Length=1

Output:

Length is: 1

Angle is: 0.8

Start point coordinates are (0; 0)

End point coordinates are (0.696707; 0.717356)

Test Case 2: Default() function

Please enter the number of segments

3

Enter angle of the new segment

0.7

Enter length of the new segment

5

Enter angle of the new segment

0.9

Enter length of the new segment

6

Enter angle of the new segment

1.2

Enter length of the new segment

6

Length is: 5

Angle is: 0.7

Start point coordinates are (0; 0)

End point coordinates are (3.82421; 3.22109)

Length is: 6

Angle is: 0.9

Start point coordinates are (3.82421; 3.22109)

End point coordinates are (7.55387; 7.92105)

Length is: 6

Angle is: 1.2

Start point coordinates are (7.55387; 7.92105)

End point coordinates are (9.72802; 13.5133)

The program will be set to default position

Length is 5

Angle to x axis is 0

Start point coordinates are (0; 0)

End point coordinates are (5; 0)

Length is 6

Angle to x axis is 0

Start point coordinates are (5; 0)

End point coordinates are (11; 0)

Length is 6

Angle to x axis is 0

Start point coordinates are (11; 0)

End point coordinates are (17; 0)

Test Case 3: Kinematics() function

Please enter the number of segments

3

Enter angle of the new segment

0.8

Enter length of the new segment

5

Enter angle of the new segment

0.6

Enter length of the new segment

9

Enter angle of the new segment

0.4

Enter length of the new segment

5

Please enter the values for angles that you wish to update your current angles to

0.9

1.2

1.4

Forward Kinematics has taken place

Length is 5

Angle to x axis is 0.9

Start point coordinates are (0; 0)

End point coordinates are (3.10805; 3.91663)

Length is 9

Angle to x axis is 1.2

Start point coordinates are (3.10805; 3.91663)

End point coordinates are (6.36927; 12.305)

Length is 5

Angle to x axis is 1.4

Start point coordinates are (6.36927; 12.305)

End point coordinates are (7.21911; 17.2322)

Test Case 4: draw() function

The program needs to be able to draw a plot of a created robot

Algorithm design:

main.cpp file

include “electrical engineering case_robot.h”

counter in class Point equals 0

counter in class Segment equals 0

counter in class Robot equals 0

main() function

declare numOfSegments as integer

declare S1(0.8, 1.0) as Segment class

print “How many segments do you want to start with?”, newline

read into numOfSegments

declare R1(numOfSegments) as Robot class

call printRobotInfo function for R1

call add function for R1

call printRobotInfo function for R1

call remove function for R1

call printRobotInfo function for R1

call Default function for R1

call printRobotInfo function for R1

call Kinematics function for R1

call printRobotInfo function for R1

print “Segment info: ”, newline

call prinS function for S1

call plot function for R1

electrical engineering case_point.h file

include iostream library

include cmath library

Class Point

private members

declare pointID as integer

declare x as double

declare y as double

declare counter as static integer

public members

initialize pointID with counter

initialize x as 0

initialize y as 0

increment counter

function getNumPoints

returns counter

function getX

returns x

function getY

returns y

end

electrical engineering case_segment.h file

include electrical engineering case_point.h file

Class Segment

private members

declare sID as integer

declare angle as double

declare length as double

declare ctr as static integer

declare start as Point class

declare end as Point class

declare childSegment as dynamic array

function endPoint

x1=x from function getX+(length from function getLength*cos(angle from function getAngle))

y1=y from function getY+(length from function getLength*sin(angle from function getAngle))

assign x1 and y1 as a Point class to end

public members

function Segment

initialize sID with counter

initialize angle with 0

initialize length with 0

initialize starting point's coordinates with 0

initialize end point with the function to calculate the end point

increment counter

function Segment

assign counter to sID

assign A to angle

assign L to length

assign 0 to coordinates of the starting point

assign getEnd function to end

increment counter

function getNumSegments

return counter

function getAngle

return angle

function getStart

return start

function setChildSegment

assign previousSegment to childSegment

function getChildSegment

return childSegment

function getSegmentLength

return length

Class Point function getEnd

function to calculate the end point

return end

function getStartX

return function getX from start Point class

function getStartY

return function getY from start Point class

function getEndX

return function getX from end Point class

function getEndY

return function getY from end Point class

function getStartXY

assign class Point with newX and newY values to start

function to calculate the end point

function getLength

return length

function getsID

return sID

function getCtr

return counter

function printS

print “The length is:”, function getLength, newline

print “The angle is:”, function getAngle, newline

print “The start point”,”(”, function getStartX, “;”, function getStartY, “)”, newline

print “The end point”,”(”, function getEndX, “;”, function getEndY, “)”, newline

function setAngle

assign A to angle

function to calculate the end point

function setLength

assign L to length

function to calculate the end point

electrical engineering case_robot.h file

include electrical engineering case_segment.h file

include pbPlots.hpp library

include supportLib.hpp library

Class Robot

private members

declare ID as integer

declare S as dynamic array

declare the number of segments as static integer

public members

Robot(numOfSegments)

assign realSegments + numSegments to realSegments

assign realSegments as a new size of the S  dynamic array

for i<realSegment times do

declare angle and length as double

print “Please, enter the length for the new segment”, newline

read into length

while length<0 do

print “Please, enter the positive value for the length for the new segment”, newline

read into length

print “Please, enter the angle for the new segment”, newline

read into angle

while angle<0 do

print “Please, enter the positive value for the angle for the new segment”, newline

read into angle

set angle in the segment class as the entered value

set length in the segment class as the entered value

set start point coordinates in the Segment class

function add

declare addition as integer

print “Please, enter the number of segments to add”, newline

read into add

while add<0 do

print “Please, enter the positive value”, newline

read into add

assign realSegments + addition to addNumSegments

declare S1 as dynamic array

assign addNumSegments as a new size of S1 array

for i < addNumSegments do

S1[i] = S[i]

Increment i

delete array of segments S

assign addNumSegments as a new size to the dynamic array S

for i < addNumSegments do

S[i] = S1[i]

Increment i

delete array of segments S1

for i < addition do

declare angle and length as double

print “Please, enter the angle for the”, i+1, “additional segment”, newline

read into angle

while angle<0 do

print “Please, enter the positive value for the angle”, newline

read into angle

print “Please, enter the length for the”, i+1, “additional segment”, newline

read into length

while length<0 do

print “Please, enter the positive value for the length”, newline

read into length

set angle in the segment class as the entered value

set length in the segment class as the entered value

set start point coordinates in the Segment class

assign number of segments with additional segments to realSegments

function remove

declare del as integer

print “Please, enter the number of segments you would like to remove”, newline

read into del

while del<0 do

print “Please, enter the positive value”, newline

read into del

assign realSegments - del to addNumSegments

declare S1 as dynamic array

assign addNumSegments as a new size of S1 array

for i < addNumSegments do

S1[i] = S[i]

Increment i

delete array of segments S

assign addNumSegments as a new size to the dynamic array S

for i < addNumSegments do

S[i] = S1[i]

Increment i

delete array of segments S1

assign new number of segments with segments to realSegments

function Default

print “The program will be set to the default position”, newline

for i < realSegments do

set angle in the Segment class to 0

set the start coordinates in the Segment class

function Kinematics

declare angle as dynamic array

assign realSegments as a new size for the dynamic array angle

declare S1 as dynamic array

assign addNumSegments as a new size of S1 array

for i < addNumSegments do

S1[i] = S[i]

Increment i

delete array of segments S

assign addNumSegments as a new size to the dynamic array S

print “Please enter the values for angles to be updated”, newline

for i < realSegments do

read into array angle

increment i

for i < realSegments do

set angle in the segment class as the entered value

set length in the segment class as the entered value

set start point coordinates in the Segment class

increment i

delete S1 array

print “Forward Kinematics has taken place”, newline

function getNumberOfSegments

return realSegments

function printRobotInfo

for i < realSegments do

call printS function

increment i

function plot

function sample is taken from internet sources, references are given in the bibliography section

Step 4: Implementation

//***********************************************************//

//** Author: Gulsim Azirakhmet                             **//

//** Date Created: December 6, 2021                        **//

//** Assignment 4: Robot Kinematics                        **//

//** UH-1000 Computer Programming for Engineers, NYUAD     **//

//** Problem:                                              **//

//** Building a Robot                                      **//

//** Using Concepts of Forward Kinematics                  **//

//***********************************************************//

#include "electrical engineering case_robot.h"

int Point::ctr = 0;

int Segment::ctr = 0;

int Robot::realSegments = 0;

int main() {

int numOfSegments;

Segment S1(0.8, 1.0);

cout << "How many segments do you want to start with?" << endl;

cin >> numOfSegments;

Robot R1(numOfSegments);

R1.printRobotInfo();

R1.add();

R1.printRobotInfo();

R1.remove();

R1.printRobotInfo();

R1.Default();

R1.printRobotInfo();

R1.Kinematics();

R1.printRobotInfo();

cout << "Segment info:" << endl;

S1.printS();

R1.plot();

}

#include <iostream>

#include <cmath>//include cmath library

using namespace std;

class Point

{

private:

int pointID;//declare pointID as integer

double x;//declare x as double

double y;//declare y as double

static int ctr;//declare counter as static integer

public:

Point()

{

pointID = ctr;//initialize pointID with ctr

x = 0;//initialize x as 0

y = 0;//initialize y as 0

ctr++;//increment counter

}

Point(double X, double Y)

{

pointID = ctr;//assign counter to pointID

x = X;//assign X to x

y = Y;//assign Y to y

ctr++;//increment counter

}

static int getNumPoints()//get number of points

{

return ctr;

}

double getX()//get x

{

return x;

}

double getY()//get y

{

return y;

}

};

#include "electrical engineering case_point.h"//include point class

class Segment

{

private:

int sID;

double angle;

double length;

static int ctr;

Point start;

Point end;

Segment* childSegment;

//function for calculating end point

void endPoint()

{

double x1 = start.getX() + (getLength() * cos(getAngle()));//calculate coordnate for x

double y1 = start.getY() + (getLength() * sin(getAngle()));//calculate coordinate for y

end = Point(x1, y1);//assign end coordinates to end

}

public:

Segment()

{

sID = ctr;//initialize sID with counter

angle = 0.0;//initialize angle with 0

length = 0.0;//initialize length with 0

start = Point(0, 0);//initialize starting point's coordinates with 0

end = getEnd();//function to calculate the end point

ctr++;//increment counter

}

Segment(double A, double L)

{

sID = ctr;//assign counter to sID

angle = A;//assign A to angle

length = L;//assign L to length

start = Point(0, 0);//assign 0 coordinates to start point

end = getEnd();//assign getEnd function to end

ctr++;//increment counter

}

static int getNumSegments()//get number of segments

{

return ctr;

}

Point getStart()//get start point

{

return start;

}

double getAngle()//get angle

{

return angle;

}

void setChildSegment(Segment* previousSegment)//set child segment

{

childSegment = previousSegment;

}

Segment getChildSegment()//get child segment

{

return *childSegment;

}

double getSegmentLength()//get length

{

return length;

}

Point getEnd()//get end point

{

endPoint();//function to calculate the end point

return end;

}

double getStartX()//get start coordinate of x

{

return start.getX();

}

double getEndX()//get end coordinate of x

{

return end.getX();

}

double getStartY()//get start coordinates of y

{

return start.getY();

}

double getEndY()//get end coordinates of y

{

return end.getY();

}

void setStartXY(double newX, double newY)//get start point coordinates

{

start = Point(newX, newY);

endPoint();

}

double getLength()//get length

{

return length;

}

int getsID()//get sID

{

return sID;

}

static int getCtr()//get the counter

{

return ctr;

}

void printS()//function to print to the output screen

{

cout << "The length is: " << getLength() << endl;//print length

cout << "The angle is: " << getAngle() << endl;//print angle

cout << "The start point " << "(" << getStartX() << "; " << getStartY() << ")" << endl;//print start point

cout << "The end point " << "(" << getEndX() << "; " << getEndY() << ")" << endl;//print end point

}

void setAngle(double A)//set angle

{

angle = A;

endPoint();//calculates end point

}

void setLength(double L)//set length

{

length = L;

endPoint();//calculates end point

}

};

#include "electrical engineering case_segment.h"//include segment class

#include "pbPlots.hpp"//include plotting library

#include "supportLib.hpp"//include support library for plotting

class Robot

{

private:

int ID;//declare ID as integer

Segment* S;//dynamic array

static int realSegments;//declare the number of segments as static integer

public:

Robot(int numSegments)

{

realSegments = realSegments + numSegments;//calculate real number of segments

S = new Segment[realSegments];//assign new size to the dynamic array

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

double angle, length;//declare angle and length as double

cout << "Please, enter the length for the new segment" << endl;//print message

cin >> length;//enter the answer to the length

while(length<0)//while value is negative do

{

cout << "Please, enter the positive value for the length for the new segment" << endl;//print message

cin >> length;//enter the answer to the length

}

cout << "Please, enter the angle for the new segment" << endl;//print message

cin >> angle;//enter the answer to the angle1

while (angle < 0)//while value is negative do

{

cout << "Please, enter the positive value for the angle for the new segment" << endl;//print message

cin >> angle;//enter the answer to the angle

}

S[i].setAngle(angle);//set angle in the segment class as the entered value

S[i].setLength(length);//set length in the segment class as the entered value

S[i].setStartXY(S[i - 1].getEndX(), S[i - 1].getEndY());//set start point coordinates

}

}

//function for the additional segments

void add()

{

int addition;//declare addition as integer

cout << "Please, enter the number of segments to add " << endl;//print message for additional segment numbers

cin >> addition;//read into additon

while (addition < 0)//while value is negative do

{

cout << "Please, enter the positive value" << endl;//print message

cin >> addition;//enter the answer to the addition

}

int addNumSegments = realSegments + addition;//new number of segments with added segments

Segment* S1;//dynamic array

S1 = new Segment[addNumSegments];//assign new size to the dynamic array

for (int i = 0; i < addNumSegments; i++)//loop that repeats number of segment times

{

S1[i] = S[i];//assign prevoius array of segments to new array

}

delete[] S;//delete array of segments

S = new Segment[addNumSegments];//assign new size to the dynamic array

for (int i = 0; i < addNumSegments; i++)//loop that repeats number of segment times

{

S[i] = S1[i];//assign S1 array to S array

}

delete[] S1;//delete S1 array

for (int i = 0; i < addition; i++)//loop that repeats additional number of segment times

{

double angle, length;//declare angle and length as double

cout << "Please, enter the angle for the " << i + 1 << " additional segment" << endl;//print the message

cin >> angle;//read into angle

while (angle < 0)//while value is negative do

{

cout << "Please, enter the positive value for the angle " << endl;//print message

cin >> angle;//enter the answer to the angle

}

cout << "Please, enter the length for the " << i + 1 << " additional segment" << endl;//print the message

cin >> length;//read into length

while (length < 0)//while value is negative do

{

cout << "Please, enter the positive value for the length" << endl;//print message

cin >> length;//enter the answer to the length

}

S[realSegments + i].setAngle(angle);//set angle in the segment class as the entered value

S[realSegments + i].setLength(length);//set length in the segment class as the entered value

S[realSegments + i].setStartXY(S[realSegments + i - 1].getEndX(), S[realSegments + i - 1].getEndY());//set start point coordinates

}

realSegments = addNumSegments;//assign number of segments with additional segments to realSegments

}

void remove()

{

int del;//declare del as integer

cout << "Please, enter the number of segments you would like to remove" << endl;//print message

cin >> del;//read value into del

while (del < 0)//while value is negative do

{

cout << "Please, enter the positive value" << endl;//print message

cin >> del;//enter the answer to the del

}

int addNumSegments = realSegments - del;//calculate number of segments

Segment* S1;//dynamic array

S1 = new Segment[addNumSegments];//assign new size to dynamic array

for (int i = 0; i < addNumSegments; i++)//loop that repeats number of segment times

{

S1[i] = S[i];//assign prevoius array of segments to new array

}

delete[] S;//delete array of segments

S = new Segment[addNumSegments];//assign new size to the dynamic array

for (int i = 0; i < addNumSegments; i++)//loop that repeats number of segment times

{

S[i] = S1[i];//assign S1 array to S array

}

delete[] S1;//delete S1 array

realSegments = addNumSegments;//assign new number of segments with segments to realSegments

}

//function to reset to default

void Default()

{

cout << "The program will be set to the default position" << endl;//print the message

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

S[i].setAngle(0);//set angle to 0

S[i].setStartXY(S[i - 1].getEndX(), S[i - 1].getEndY());//set the start coordinates

}

}

//function to perform kinematics

void Kinematics()

{

double* angle;//dynamic array

angle = new double[realSegments];//assign new size to dynamic array

Segment* S1;//dynamic array

S1 = new Segment[realSegments];//assign new size to dynamic array

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

S1[i] = S[i];//assign prevoius array of segments to new array

}

delete[] S;//delete S array

S = new Segment[realSegments];//assign new size to dynamic array

cout << "Please enter the values for angles to be updated " << endl;//print the message

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

cin >> angle[i];//read into angle array

}

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

S[i].setAngle(angle[i]);//set read angles

S[i].setLength(S1[i].getLength());//set length

S[i].setStartXY(S[i - 1].getEndX(), S[i - 1].getEndY());//set start point coordinates

}

delete[] S1;//delete S1 array

cout << "Forward Kinematics has taken place" << endl;//print message

}

static int getNumberOfSegments()//fumction to get a number of segments

{

return realSegments;

}

void printRobotInfo()//function to print robot info

{

for (int i = 0; i < realSegments; i++)//loop that repeats number of segment times

{

S[i].printS();//call printS

}

}

//function to plot the graph

void plot()//function sample is taken from internet sources, references are given in the lab report

{

vector<double> xs;

vector<double> ys;

for (int i = 0; i < realSegments; i++)

{

xs.push_back(S[i].getStartX());

xs.push_back(S[i].getEndX());

ys.push_back(S[i].getStartY());

ys.push_back(S[i].getEndY());

}

bool success;

StringReference* errorMessage = new StringReference();

RGBABitmapImageReference* imageReference = CreateRGBABitmapImageReference();

success = DrawScatterPlot(imageReference, 600, 400, &xs, &ys, errorMessage);

if (success)

{

vector<double>* pngdata = ConvertToPNG(imageReference->image);

WriteToFile(pngdata, "example1.png");

DeleteImage(imageReference->image);

}

else

{

cerr << "Error: ";

for (wchar_t c : *errorMessage->string)

{

wcerr << c;

}

cerr << endl;

}

}

};

Step 5: Software Testing and Verification

Test Case 1: printS() function

Output:

Test Case 2: Default() function

Test Case 3: Kinematics() function

Test Case 4: plot() function

All the data in the test cases are logical and are in agreement with the expectations

User’s Guide

This program will help you to identify the coordinates of the last point of the robot’s segment. You will be asked to enter the angle and length for the each segment of the robot. You may add or remove the segments of the robot. Program will show you the coordinated of the last point and plot a graph of the robot’s segments. Program has functions to set the point to the default position to set angles to 0 and kinematics function to change angles.

Bibliography

“An easy way to plot in a Visual Studio 2019 C++ program, ” Microsoft Q&A, September 27, 2021, https://docs.microsoft.com/en-us/answers/questions/567439/an-easy-way-to-plot-in-a-visual-studio-2019-c-prog.html.